# System Overview

The Intelligent Solar-Powered Vendo-Washing Machine integrates four main subsystems that work together seamlessly: solar power generation, energy storage and distribution, embedded control electronics, and the washing machine unit. The solar panel captures sunlight and charges the battery through a controller, while breakers and an inverter manage safe distribution of power to both the control circuits and the washing machine. The embedded system, built around an Arduino UNO, handles user interaction via the coin slot, LCD display, and start button, while relay modules switch the washing machine and other components on and off.

When a user inserts a coin, the vending mechanism validates payment and communicates with the Arduino, which then enables the washing cycle. The LCD provides real-time feedback, and the relays ensure proper activation of the washing machine. Together, these subsystems create a fully automated, energy-efficient laundry solution that operates independently of the grid, offering both sustainability and convenience.

The Intelligent Solar-Powered Vendo-Washing Machine Using Embedded Systems is a sustainable and automated laundry solution designed to operate independently using solar energy. This system integrates renewable power generation, embedded control, and vending functionality to provide an efficient, eco-friendly washing experience.

Powered by a 12V DC solar array, the system converts sunlight into electrical energy stored in a deep-cycle battery bank. Through a solar charge controller, the energy flow is regulated to ensure safe and optimal charging. The stored energy is then distributed to both the control electronics (Arduino-based automation) and the washing machine via a DC-to-AC inverter, enabling standard 220V AC operation.The embedded system, built around an Arduino UNO, manages user interaction and payment validation through an Allan Universal Coin Slot. Once a valid coin is inserted, the system activates the washing cycle using a relay module and displays real-time status updates on a 20×4 LCD screen. A Start button allows users to initiate the process, while safety breakers and regulated power lines ensure reliable operation.

This manual provides a step-by-step guide for assembling, wiring, and operating the system—from solar setup to embedded control integration. Each section includes detailed descriptions, wiring instructions, and testing procedures to help you build and maintain a fully functional, intelligent, and energy-efficient vendo-washing machine.

## ⚙️ Step 1: Solar Power Setup

Components:

- Solar Array (1×100W panel)

- Solar Charge Controller

- 12V Battery Bank (Deep Cycle Lead-Acid)

Instructions:

- Mount the Solar Panel:

  - Place the panel in direct sunlight, ideally facing south (in the Philippines) at a 15–20° tilt.

  - Ensure no shading during peak hours (9 AM–3 PM).

- Connect to Charge Controller:

  - Connect Solar + and Solar – wires from the panel to the SOLAR IN terminals on the charge controller.

  - Use thick gauge wires (≥10 AWG) for minimal voltage drop.

- Connect Battery Bank:

  - Attach BATTERY + OUT and BATTERY – OUT from the controller to the battery terminals.

  - Confirm polarity before tightening connections.

  - The controller will regulate charging automatically.

- Grounding:

  - Connect the solar panel frame and controller ground to earth for safety.

## ⚡ Step 2: DC Power Distribution

Components:

- DC Main Breaker (100A)

- DC Distribution Breaker (60A)

- 12V DC to 220V AC Inverter

Instructions:

- Install Breakers:

  - Place the Main Breaker between the battery and inverter.

  - Place the Distribution Breaker between the battery and control circuit.

- Connect Inverter:

  - Connect DC + and DC – from the battery output to the inverter’s DC input.

  - Connect the inverter’s 220V AC OUTPUT to the washing machine’s power line.

- Safety Check:

  - Ensure breakers are rated correctly and securely mounted.

  - Test continuity before powering on.

## 🔋 Step 3: Control Power Supply

Components:

- 12V DC Power for Controls

- Arduino UNO

- 4-Relay Module

- 20×4 LCD Display

- Start Button

Instructions:

- Power the Arduino:

  - Connect the 12V DC Power for Controls to the Arduino’s VIN and GND pins.

  - Verify the onboard LED lights up.

- Connect LCD Display:

  - Use the I2C interface (SDA and SCL pins) to connect the LCD to the Arduino.

  - Upload a test sketch to display “SYSTEM: ARMED”.

- Wire the Start Button:

  - Connect NO (Normally Open) to Arduino digital input.

  - Connect LED terminal to 5V and GND for illumination.

  - Program the button to trigger washing cycle start.

- Relay Module Connection:

  - Connect relay IN1–IN4 to Arduino digital pins (e.g., D7–D10).

  - Each relay controls a specific load (e.g., coin slot, washing machine power, water valve, etc.).

  - Ensure relay VCC = 5V and GND = Arduino GND.

## 💰 Step 4: Coin Slot Integration

Components:

- Allan Universal Coinslot

- DC Female Adapter

Instructions:

- Connect Coin Slot:

  - Plug the DC Female Adapter into the coin slot’s power input.

  - Connect signal output from the coin slot to Arduino digital pin (e.g., D2).

- Program Coin Detection:

  - Configure Arduino code to detect coin insertion.

  - When a valid coin is inserted, display “Credit Accepted” on the LCD and enable the Start button.

- Test Functionality:

  - Insert a coin and verify the LCD updates and relays activate accordingly.

## 🧺 Step 5: Washing Machine Connection

### Components:

- Portable Washing Machine

- AC Mains Power (220V)

### Instructions:

- Connect AC Output:

  - From the inverter’s 220V AC OUTPUT, connect to the washing machine’s power input.

  - Use proper insulation and rated plugs.

- Relay Control:

  - One relay should act as the power switch for the washing machine.

  - Arduino logic should activate this relay when the system is armed and the Start button is pressed.

- Safety Precautions:

  - Ensure all AC connections are insulated.

  - Never touch wiring while the system is powered.

## 🧠 Step 6: System Testing and Operation

Instructions:

- Initial Power-Up:

  - Turn on the DC breakers.

  - Observe the charge controller display for battery voltage and charging status.

- Arduino Boot Sequence:

  - LCD should display “SYSTEM: ARMED”.

  - Coin slot should be ready to accept coins.

- Operation Flow:

  - Insert coin → LCD shows “Credit Accepted”.

  - Press Start → Relay activates washing machine.

  - Washing cycle runs for programmed duration.

  - System resets after completion.

- Maintenance:

  - Clean solar panels monthly.

  - Check battery voltage regularly.

  - Inspect wiring and breakers for corrosion or looseness.

## 🧾 Step 7: Troubleshooting Guide

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| LCD not displaying | Loose I2C connection | Recheck SDA/SCL wiring |
| No power from inverter | Battery low or breaker off | Charge battery, reset breaker |
| Coin not detected | Faulty coin slot or signal wire | Test with multimeter, replace slot |
| Washing machine not starting | Relay not switching | Check relay wiring and Arduino code |